«АТАКТIКА» Ltd

St. Kosareva, 8, office 54, Tomsk, Russia, 634012

Tel/fax: +7 (3822) 55-87-60

Mob.+7-960-972-31-47

e-mail: vtv752@yandex.ru   www.ataktika.narod.ru

Oxidized ataxic polypropylene (ОАPP)

Oxidized ataxic polypropylene is designed for bitumen polymer compound materials, structural mastic, anticorrosion primers, coatings and fillings, water-resistant and anti-icing coatings, polymer-polymer and polymer-mineral compositions, glues for scotch tape and melting glue that are used in building works and petrochemical and mechanical rubber industries as multi-functional additives to lubrication oil, diesel fuel (depressant) and to rubber mixtures. OAPP is a new product with unique structure and necessary operational properties which origin non-modified APP doesn’t have. OAPP has olefinic double bounds, polar hydroxyl and carbonyl groups in macro molecular. Their quantity could be easily controlled by the conditions of oxidation reaction which is resulted in various products. OAPP highly reacts with poly bounded compounds contained in ready bitumen that allows preparing bitumen polymer compounds at the temperature up to 159oC and increasing durability period of bitumen compositions in 2-3 times. OAPP has high adhesion properties to polar materials. OAPP is used for producing high-quality compositions for lubricating materials, anticorrosion composites and polymer compounds. OAPP is well dissolved in oil products and contains considerably low number of admixtures of non-dissolved isotactic fraction. OAPP is also applied in hermetic composites for building works, it is a necessary ingredient which could partly replace expensive caoutchouc and polyisobutylene and keep necessary qualities of hermetics. OAPP is a hydrophobic polymer despite it contains high level of carbonyl groups which are applied for producing melting glues. The cost of a tone of caoutchouc is 35000-60000 rubles; polyisobutylene – 100000-120000 rubles; OAPP is about 53000 rubles, depending on brand and price of row materials.

Technical specification for three brands of OAPP has been made.

«АТАКТIКА» Ltd produces tonnage volumes of OAPP.

OAPP advantages:

- multifunctional of new material under considerably low price

- increasing in heat firmness of modified goods 1,3-1,6 times

- advanced hydrophobic, adhesion, viscous, antioxidized qualities of compound materials modified with OAPP

- increasing in work period 2-3 times of goods modified with OAPP

Cooperation proposal:

- signing of contracts aimed at oxidized ataxic polypropylene supply

- cooperation in the sphere of production of oil, lubrications, bitumen, roofing materials with advanced chemical and technical characteristics

- development of new polymer-polymer and polymer-mineral compositions with the usage of OAPP

Contacts: Tatiana V. Vedernikova,

tel./fax +7(3822) 55-87-60

e-mail vtv752@yandex.ru,